Disclaimer

THIS SPECIFICATION IS LICENSED AND PROVIDED BY LOGITECH "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY ANDFITNESS FOR A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL LOGITECH OR ANY OF ITS AFFILIATED COMPANIES BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE OF THIS SPECIFICATION, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

DRAFT DOCUMENT

The feature has 3 functions and 1 events:

[0x6100]TouchPadRawXY

GetTouchPadInfo	=  [0]GetTouchPadInfo()

GetRawReportState	=  [1]GetRawReportState()

void                    	=  [2]SetRawReportState(uint8_t state)

[0x6100] EVENT TouchPadRawXY

DualXYData 		=  [0]TwoFingersDataReport()

[0x6100] FUNCT GetTouchpadInfo()

Summary

Returns the TouchPad characteristics (size, number of dots, data ranges, etc)

Parameters

none

Returns

| byte num (in payload) | description | remarks |
| --- | --- | --- |
| 0 | X size (dots) H | max pixels reported MSB |
| 1 | X size (dots) L | max pixels reported LSB |
| 2 | Y size (dots) H | max pixels reported MSB |
| 3 | Y size (dots) L | max pixels reported LSB |
| 4 | Z data range (size) | 0x00 means no range 0x0F means 16bits |
| 5 | Area data range (size) | 0x0F means 16bits |
| 6 | Timestamps unit given in 0.1ms | A value of 10 means timestamps are in units of 1 mSec See Note 1) |
| 7 | Max number of fingers |  |
| 8 | Position of the origin | 0x00 = reserved 0x01 = LOWER-LEFT 0x02 = LOWER-RIGHT 0x03 = UPPER-LEFT 0x04 = UPPER-RIGHT Note: corners are defined by looking at device from above, with lower edge toward the user and upper facing the PC screen |
| 9 | RESERVED |  |
| 10 | RESERVED |  |
| 11 | RESERVED |  |
| 12 | RESERVED |  |
| 13 | DPI_H | Touchpad Resolution MSB (Same resolution in X and Y) |
| 14 | DPI_L | Touchpad Resolution LSB (Same resolution in X and Y) |
| 15 | RESERVED |  |

Note 1) Exception: A device that reports 8 in this field has a timestamp unit of 1ms (i.e field value equivalent to 10). There is no support for devices with 0.8 mSec timestamp unit.

Errors None

[0x6100] FUNCT GetRawReportState

Summary

Returns the TouchPad raw reporting state byte.

Parameters

none

Returns

| bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD | RSVD | RSVD | raw and native | width/height 4-bit reporting | enhance | add force data on 16bits | enable raw |

Byte 0: State

[0x6100] FUNCT SetRawReportState

Summary

Sets the TouchPad raw reporting state byte.

Parameters

Byte 0: state

Returns

| bit 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD | RSVD | RSVD | eaw and native | width/height 4-bit reporting | enhance | add Z info on 16bits | enable raw |

none

Reporting state details:

bit0: enable raw reporting

0 = DISABLED (Touchpad reports on standard HID pipe. Gestures are processed in the touchpad)

1 = ENABLED: (Touchpad reports raw data on HIDPP pipe, nothing is sent on the standard HID pipe. No gesture processing is done by the touchpad)

bit1: (add Z info on 16bits)

0 = DISABLED (Area byte in report contains the contact area. Z is reported as 8 bit value)

1 = ENABLED (Area byte in report contains the MSB of a 16bit Z value. No area is reported)

bit2: - Enhance

0 = DISABLED (Normal settings for sensor)

1 = ENABLED (Enhanced sensibility for sensor)

bit3: - Width/Height reporting (4-bit each) instead of Area

0 = DISABLED

1 = ENABLED (report width/height)

bit4: - Send RAW data as well as recognized gestures/tracking/scrolling messages. (dual mode)

0 = DISABLED

1 = ENABLED (send both, raw data on hidpp pipe and standard data on hid channel) This mode may degrade the tracking smoothness.

Returns

none

EVENT [0] format:

[0x6100] Event TouchPadRawTouchPoints

Summary

Sends data for two fingers, out of the N currently used. Sent by chunks of 2 fingers.

Parameters

none

Returns

DualXYData 	[16 x 8bits]

| byte num (in payload) | description | covers |
| --- | --- | --- |
| 0 | Timestamp (MSB) | tags this packet (both touch points) |
| 1 | Timestamp (LSB) |  |
| 2 | 2b Contact type 6b X (MSB) | touch point 1 |
| 3 | X (LSB) |  |
| 4 | 2b Contact status 6b Y (MSB) |  |
| 5 | Y (LSB) |  |
| 6 | Z/Force |  |
| 7 | Area or (width + height) |  |
| 8 | 4b Finger ID 1b proximity detection 1b mechanical button 1b Spurious Flag 1b End-of-Frame (*) |  |
| 9 | 2b Contact type 6b X (MSB) | touch point 2 |
| 10 | X (LSB) |  |
| 11 | 2b Contact status 6b Y (MSB) |  |
| 12 | Y (LSB) |  |
| 13 | Z/Force |  |
| 14 | Area |  |
| 15 | 4b Finger ID 4b Total number of fingers in this frame |  |

CONTACT TYPE:

00 = finger

01 = reserved

10 = reserved

11 = reserved

CONTACT STATUS (2bits)

00 = no finger (released)

01 = touch

10 = hover

11 = reserved

Note: in order to release finger the device will send an empty report (ie with all data to 0).

It is important to note that X=0, Y=0, Force=0, Area=0 are still valid values, therefore the host software (driver) should check for CONTACT_STATUS = 0x00 to assess the release of that finger.

In case more than 2 fingers are present, two or more event will be sent to complete the frame:

The timestamp will be the same (it represents the timestamp of the moment when the frame is captured)

The total number of fingers (in the frame) is repeated in all messages of the frame

The last message of the frame will be "tagged" with the 1b End-of-Frame bit set to 1.